CyberDisplayTM

KCD-MQ01-AC

CyberDisplay 320 Monochrome RS170 Module

The CyberDisplay 320 Monochrome RS170 Module interfaces the CyberDisplay 320 Module 290 Monochrome Display to a monochrome RS170 video input.  The module contains all the circuitry necessary to process, format, and display RS170 video on the 320 x 240 monochrome display.  It also allows users to interface to a composite video input by adding the color filter onto the board.  The list below shows the features available which allow users to customize the module:

- On Screen Display

- Color Filter

- Sleep Mode

- NTSC or PAL

- Contrast Adjustment

- Brightness Adjustment

- Gamma Adjustment

The board will operate with a 2.7V to 5.5V power supply (not included).  It can also be powered by three 1.5V batteries( not included). The board is approximately 1 inch x 2 inches and weighs 9 grams.

The diagram below shows the connections for video input and power for the module as well as the backlight connections.  The display is placed in the connector with the contacts down.

| 1 | Vdd In (Opt) |
| --- | --- |
| 2 | Power In |
| 3 | Digital Gnd |
| 4 | Video In |
| 5 | Analog Gnd |

Connector J4

Exceeding 5.5V input may cause damage to the module and/or the display.

K o p i n  C o r p o r a t i o n

695 Myles Standish Blvd., Taunton, MA 02780

T 508.824.6696, F 508.822.9018

9/12	©  2000 	KLQD-011